For Poster Presentation Session Only – Poster Content Already Approved for Public Release

Minimum-Time Low-Thrust Transfer from a Distant Retrograde Orbit to a Tulip-Shaped Orbit for Lunar PNT

Mike Casey, PhD and Darin Koblick, PhD

darin@coorbital.com

Coorbital Inc. Rolling Hills Estates, CA

Recently discovered tulip-shaped cislunar orbits offer favorable navigation geometry over the Moon's southern hemisphere, where sustained surface operations are planned. We address the  deployment logistics of moving a navigation and relay satellite from a distant retrograde orbit (DRO) into an operational tulip-shaped orbit under continuous low thrust. A DRO is an attractive staging location because it is long-term stable and remains accessible from Earth across a wide range of launch dates and is already baselined for cislunar logistics. It also allows an entire constellation to be delivered on a single flight and released incrementally.

We model a 150 kg spacecraft with a 70 mN Hall Effect thruster and a specific impulse of 900 seconds in the Earth-Moon Circular Restricted Three-Body Problem (CR3BP). The minimum-time transfer is solved by indirect optimal control, in which Pontryagin's Minimum Principle converts the problem into a two-point boundary value problem. Convergence is obtained by [continuation from a higher-thrust solution], and the resulting trajectory satisfies the first-order necessary conditions of optimality. The transfer takes approximately 18 days and requires about 0.75 km/s of ΔV, or 12.2 kg of propellant, roughly eight percent of the wet mass, at an average thrust acceleration near 0.5 mm/s².

Because the CR3BP is time invariant, a single transfer solution can be shifted in epoch and reused. Every spacecraft therefore flies an identical trajectory, and a staggered schedule of departures from the shared staging orbit places each vehicle in a distinct phase slot on the same 7-petal 6:5 resonant tulip-shaped orbit. We adopt the optimized six-vehicle phasing from our recent work on tulip-orbit constellations for lunar south pole navigation and direct-to-Earth relay (arXiv:2607.10482), so each spacecraft arrives directly in its operational slot. Deployment reduces to one trajectory design and a phased release schedule, with the full six-satellite constellation reaching its operational configuration [X] days after the first departure.

Sustainment of the target orbit is treated separately in our station-keeping analysis of this family, which shows annual maintenance below 15 m/s (AIAA 2026-1455). Extension to a full ephemeris model, including solar radiation pressure and eclipse-driven thrust outages, is the subject of ongoing work. Our findings show that a southern hemisphere PNT constellation can be deployed from a single cislunar staging orbit using low thrust, with each satellite reaching station in under three weeks.